# WT EXPERIMENT - 4

| Name | Class | Roll no. |
| --- | --- | --- |
| Chirag Choudhary | D15A | 10 |
| Manorath Ital | D15A | 19 |

#### Aim: Component and Arduino Introduction Introduction:

The Arduino Uno is a versatile and widely used microcontroller board that serves as the foundation for many electronics projects. It is an open-source platform that allows users to design and develop interactive electronic devices. The board consists of an Atmega328 microcontroller that can be programmed to control various hardware components, making it ideal for applications in automation, robotics, IoT, and more.

In this experiment, we will use an Arduino Uno to create an object segregator that differentiates between wet and dry objects using an ultrasonic sensor and a soil moisture sensor. The main goal is to measure the distance of an approaching object, analyze its moisture content, and trigger a servo motor to sort the object accordingly.

#### Arduino Uno Overview

The Arduino Uno is one of the most popular boards in the Arduino family due to its simplicity, low cost, and ease of use. It features:

- Microcontroller: Atmega328P

#### Operating Voltage: 5V

- Digital I/O Pins: 14 (6 can be used as PWM outputs)

#### Analog Input Pins: 6

- Flash Memory: 32 KB

- Clock Speed: 16 MHz

- USB Connection: For programming and communication

- Power Supply: Can be powered via USB or an external power supply

The Arduino Uno can be programmed using the Arduino IDE, which allows users to write and upload code that controls various electronic components connected to the board.

# Installation and Setup

### Installing the Arduino IDE

- Download and install the Arduino IDE from Arduino's official website.

- Connect the Arduino Uno to your computer via USB.

- Open the Arduino IDE and select the correct board under Tools > Board > Arduino Uno.

- Select the correct port under Tools > Port.

- Upload a sample program to verify the setup.

Running a Test Blink Program void setup() { pinMode(LED_BUILTIN, OUTPUT);

}

void loop() { digitalWrite(LED_BUILTIN, HIGH); delay(1000); digitalWrite(LED_BUILTIN, LOW); delay(1000);

}

If the built-in LED blinks, your setup is working fine. Components Used

- Arduino Uno: Acts as the main controller for reading sensor data and controlling the servo motor.

- Ultrasonic Sensor (HC-SR04): Measures the distance of an object.

- Soil Moisture Sensor: Determines whether the detected object is wet or dry.

- Servo Motor (SG90 or similar): Moves objects based on classification.

- Jumper Wires: Connects the components.

- Breadboard: Provides a non-permanent circuit connection.

## Code:

#include <Servo.h>

// Pin definitions const int trigPin = 7; const int echoPin = 8;

const int servoPin = 9; const int moisturePin = 2;

const int moistureThreshold = 500; Servo myServo;

void setup() { Serial.begin(9600); pinMode(trigPin, OUTPUT); pinMode(echoPin, INPUT); myServo.attach(servoPin); myServo.write(100); delay(1000);

}

void loop() {

long duration = measureDistance(); long distance = duration * 0.0344 / 2;

Serial.print("Distance: "); Serial.print(distance); Serial.print(" cm");

if (distance < 20) { delay(2000);

bool isWet = digitalRead(moisturePin);

Serial.print(", Moisture: "); Serial.print(isWet); Serial.print(" -> ");

Serial.println(isWet ? "Wet" : "Dry"); if (isWet) {

Serial.println("Wet object detected! Moving clockwise."); myServo.write(180);

} else {

Serial.println("Dry object detected! Moving counterclockwise."); myServo.write(20);

}

delay(1500);

Serial.println("Returning to 100 degrees."); myServo.write(100);

delay(2000);

} else {

Serial.println(" - No object detected. Servo remains at 100 degrees.");

}

delay(2000);

}

long measureDistance() { digitalWrite(trigPin, LOW); delayMicroseconds(2); digitalWrite(trigPin, HIGH); delayMicroseconds(10); digitalWrite(trigPin, LOW); return pulseIn(echoPin, HIGH);

}

## Code Breakdown

### Libraries

- #include <Servo.h>: Includes the Servo library to control the servo motor.

### Pin Setup

- Ultrasonic Sensor: trigPin (7), echoPin (8)

- Servo Motor: servoPin (9)

- Moisture Sensor: moisturePin (2)

### Setup Function

- Initializes the serial monitor, ultrasonic sensor, and servo motor.

- Moves the servo to the neutral position (100 degrees).

### Loop Function

- Measures the object’s distance using measureDistance().

- If an object is detected within 20 cm:

  - Reads the moisture sensor.

  - If wet, moves the servo clockwise (180 degrees).

  - If dry, moves the servo counterclockwise (20 degrees).

  - Returns the servo to its initial position.

- If no object is detected, the servo remains at 100 degrees.

### Distance Measurement Function

- Sends a pulse from the ultrasonic sensor and calculates the time taken for the echo.

- Converts this time into a distance value.

# Methodology:

- Distance Measurement Analysis:

- Once an object is detected, the ultrasonic sensor calculates the distance by measuring the time taken for the sound wave to travel to the object and back.

- The measured distance is then processed and displayed on the LCD screen in real-time.

- Distance Measurement Display:

- If the measured distance is within a valid range (e.g., 2–400 cm), the LCD displays the distance in centimeters with two decimal places.

- If the object is out of range, the LCD shows an "Out of range!" message to indicate an invalid measurement.

- Arduino Processing: The Arduino collects sensor data, processes it, and calculates the distance accordingly. It also provides real-time results with 2 seconds delay on the LCD screen

## Conclusion

This project demonstrates how Arduino can be used to classify objects based on their moisture content. By integrating multiple sensors and actuators, this system provides a simple yet effective way to automate object segregation. This fundamental concept can be extended to industrial sorting applications and environmental waste management systems.