WT Experiment 6

Topic: Wireless Temperature and Humidity Monitoring System

| Class | Roll No. | Name |
| --- | --- | --- |
| D15A | 10 | Chirag Choudhary |
| D15A | 19 | Manorath Ital |

Aim:

Study of Ultrasonic Sensor and LCD interfacing with Arduino

## 1. Introduction

## Distance measurement is an essential part of many embedded and automation systems such as robotic navigation, obstacle avoidance, and industrial safety. This project focuses on using an ultrasonic sensor (HC-SR04) to measure the distance of an object from the sensor and display the results in real time on a 16x2 I2C LCD display using an Arduino Uno. The sensor calculates the distance by measuring the time it takes for an ultrasonic pulse to travel to the object and back.

## 2. Materials and Components

## Microcontroller: Arduino Uno

## Sensor: HC-SR04 Ultrasonic Sensor

## Display: 16x2 I2C LCD Module

## Connecting Components:

## Breadboard

## Jumper Wires

## Power Supply: USB Cable or 9V battery with adapter

## 3. System Architecture

## The system consists of the following:

## HC-SR04 Ultrasonic sensor to measure distance based on the time-of-flight of ultrasonic waves.

## Arduino Uno to trigger the sensor, read echo response, compute the distance and control LCD display.

## 16x2 I2C LCD display to show the measured distance in centimeters.

## The LCD is connected to Arduino using I2C protocol to reduce wiring complexity and improve reliability.

## 4. Circuit Diagrams and Wiring

Ultrasonic Sensor (HC-SR04):

VCC → 5V

GND → GND

Trig → Digital Pin 3 on Arduino

Echo → Digital Pin 2 on Arduino

16x2 I2C LCD:

VCC → 5V

GND → GND

SDA → A4 (Arduino Uno)

SCL → A5 (Arduino Uno)

## 5. Software Implementation

Code:

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2); // Format -> (Address, Width, Height)

#define echoPin 2  // attach pin D2 Arduino to Echo pin of Sensor module

#define trigPin 3  // attach pin D3 Arduino to Trig pin of Sensor module

long duration;  // Declare variable to store echo time duration

float distance; // Declare variable to store the result (distance) as a decimal

void setup()

{

lcd.init();         // Initialize the LCD

lcd.backlight();    // Turn on the backlight

pinMode(trigPin, OUTPUT);  // Sets the trigPin as an OUTPUT

pinMode(echoPin, INPUT);   // Sets the echoPin as an INPUT

// Serial Communication is starting with 9600 baudrate speed

Serial.begin(9600);

// The text to be printed in serial monitor

Serial.println("Distance measurement using Arduino Uno");

delay(500);

}

void loop()

{

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration * 0.0344 / 2.0; // Ensure floating point division

// Print the result to the serial monitor

Serial.print("Distance: ");

Serial.print(distance, 2); // Print with 2 decimal places

Serial.println(" cm");

// Display the result on the LCD

lcd.clear();             // Clear the display buffer

lcd.setCursor(0, 0);     // Set cursor for "Distance:" (Column, Row)

lcd.print("Distance:");   // Print "Distance:" at (0, 0)

lcd.setCursor(0, 1);     // Set cursor for output value (0, 1)

lcd.print(distance, 2);   // Print the distance with 2 decimal places

lcd.setCursor(8, 1);     // Move cursor to (8, 1)

lcd.print("cm");         // Print "cm" at (8, 1)

delay(2000);

}

## 6. Testing and Calibration

## Sensor Validation: Compare the sensor readings with a standard ruler or measuring tape to verify accuracy.

## LCD Display: Ensure correct I2C address is used and the LCD shows stable readings without flicker or delay.

## 7. Applications and Future Enhancements

- Applications:

  - Robotic Obstacle Detection

  - Parking Assist Systems

  - Industrial Object Detection and Sorting

- Future Enhancements:

  - Use of servo motors to act upon specific distance ranges.

  - Data logging with SD card or cloud.

  - Bluetooth or Wi-Fi module to monitor distance remotely.

## 8. Conclusion

This project provides a simple and effective method for measuring distances using the HC-SR04 ultrasonic sensor and Arduino Uno. The integration of an I2C 16x2 LCD allows for clear, real-time display of the measurements. The setup is ideal for educational purposes, prototyping, and small-scale automation tasks.